FL1.0. 	Category 	Junior and Senior – Non-Gearbox

Class 	Formula Libre

Contact 	Motorsport UK

FL1.1. 	Introduction. A class to cater for engines that no longer have their own approved class homologations.

The only championships permitted will be single venue club championships. Drivers must be in possession of a

Motorsport UK Competition licence minimum Kart Interclub. Note for drivers under the age of 18, a Kart PG

Entrants licence is required. Organisers may make further restrictions, for example on engines or tyres, in their Supplementary Regulations with Motorsport UK approval. References in these regulations to class regulations for specific equipment means the most recent set of Motorsport UK published class regulations (frozen at the point of expiration of class homologation), unless otherwise detailed in the Kart Technical area of the Motorsport UK website (www.motorsportuk.org/resource-centre). Kart, engines, tyres and racewear must comply with Motorsport UK NCR and must be passed as safe and fit for purpose by the scrutineer.

FL1.2. Chassis. Any chassis conforming to Motorsport UK NCR for non-gearbox karts

FL1.2.1 	Rear Wheel Protection. Pre 2022 CIK-FIA homologated Rear Wheel Protection may be used in accordance with the 2021 CIK-FIA regulations.

FL1.3. 	Engine. Engines must be currently – or have been previously – approved for use in Motorsport UK short-circuit karting and remain in that approved specification, from the following list:

| Junior: | Senior: |
| --- | --- |
| Rotax Junior non-Evo or Evo | Rotax Senior non-Evo or Evo |

FL1.3.1. 	Exhaust, Inlet and Throttle Restrictors. Restrictors as detailed in the relevant fiche or approval documentation must be in place at all times and must be as supplied by the engine manufacturer.

FL1.3.2. 	Carburettor. As per class regulations for the engine, or separately approved by Motorsport UK.

FL1.3.3. 	Fuel Pump. Free

FL1.3.4. 	Intake Silencer. As per class regulations for the engine, any type currently or previously approved for the class is eligible.

FL1.3.5. Exhaust System. As per class regulations for the engine, any type currently or previously approved for the class is eligible. Vevey exhausts must have an end cap.

Karts must at all times comply with current Motorsport UK NCR noise requirements.

FL1.3.6. 	Radiator (where applicable). As per class regulations for the engine, any type currently or previously approved for the class is eligible.

FL1.3.7. 	Ignition Unit. As per class regulations for the engine, any type currently or previously approved for the class is eligible.

FL1.3.8. 	Spark Plug. As per class regulations for the engine, any type currently or previously approved for the class is eligible.

FL1.4. 	Transmission. Direct from the engine to the rear axle via a single length of chain. The clutch must be as supplied as per class regulations for the engine; any type currently or previously approved for the class is eligible.

| FL1.5. | Brakes. Hydraulic disc brake operating on rear axle only, in conformance with Motorsport UK NCR. |
| --- | --- |
| FL1.6. | Tyres. Dry: Junior - as per current 2026 Gold book for the Rotax Junior class Dry: Senior - as per current 2026 Gold book for the Rotax Senior class |
|  | Wet: Junior - as per current 2026 Gold book for the Rotax Junior class Wet: Senior - as per current 2026 Gold book for the Rotax Senior class |

If so marked, tyres must be fitted to run in the correct direction of rotation, as indicated by the arrow on the sidewall of the tyre.

FL1.7. 	General. An ignition kill switch must be fitted to any clutched engine and must be identified with an electric flash symbol sticker or marked on/off to assist marshals in the event of an incident.

FL1.7.1. 	Weight.

Junior: Minimum of 145kg including driver with a minimum driver weight, as per NCR Ch.18 Art.11.3 of 40kg.

Senior: Minimum of 162kg including driver. Minimum driver weight for any driver under the age of 16 as per NCR Ch.18 Art.12.1 is 54kg.

| FL1.7.2. | Number Plates. White with black numbers. NCR Ch.18 App.8 Art.30 applies. |
| --- | --- |
| FL1.7.3. | Age. Juniors: Calendar year of the 13th birthday to 31st December of the calendar year of the 16th birthday. Drivers who have not reached their 13th birthday must provide evidence that they have held (or have been eligible to hold) a Kart National licence for at least 12 months prior to competing in this class. Exceptionally, where an organiser restricts the class to a single engine, the ages as detailed in the applicable class regulations may be applied. |
|  | Seniors: As per Motorsport UK NCR. |

MARIA REDDY

Championship Co-ordinator     			Date 19.02.2026